FEATURES :: SUPERCHARGER TECH Superchargers A-Z (Part 3 of 3) 3/26/2003 6:35:00 PM

Welcome to the 3rd and final installment of "Superchargers A-Z". If you haven't already read Part 1 and Part 2, of this series, you may want to start there.

So far in this series we've discussed what a supercharger is, where it came from, and what technolgies drive the core of any supercharger system - the supercharger itself. Today we'll take a look at the supercharger system as a whole. Because of the radical performance differences between a supercharged engine and a normally aspirated engine, the supercharger must integrate with other critical engine systems like the ignition system and the fuel delivery system. Don't worry, though, because almost all of the supercharger systems sold today are complete supercharger systems and do not require the addition of 3rd party fuel and ignition components. With this in mind, let's break a supercharger system down into its main functional components - a discussion of the supercharger itself is not included in this article because it was the focus of part 2 of this series. Keep in mind that each supercharger system is designed for a specific application, and the specific contents of different supercharger systems vary greatly.

An example of a complete supercharger system.

The Air Intake System
Because a supercharged engine draws substantially more air than a normally aspirated engine, it is important to minimize intake restrictions. To ensure a smooth delivery of air to the supercharger, most supercharger systems include a high-flow air filter as well as low-restriction tubing or ducting to deliver air from the atmosphere to the supercharger. It is important to maintain a clean air filter to minimize the particles that come into contact with the supercharger's impeller, rotors, or screws. Most supercharger systems will draw air from behind the fender wall, where there is an abundance of cool air that has not been heated by the engine. Because superchargers heat air as it is compressed, a cool air supply helps to keep the charge temperatures at a reasonable level. On a non-intercooled application, the cold air pickup can lower the charge temperature by up to 60 degrees!

On most vehicles the incoming air charge passes through a Mass Air Flow sensor (aka MAF) on its way to the supercharger, although on centrifugal superchargers, the Mass Air Flow sensor can be mounted after the supercharger ("blow-through" setup). The Mass Air Flow sensor measures, you guessed it, the mass of air that the engine is drawing. This reading allows your engine's ECU (Electronic Control Unit) to calibrate and deliver the appropriate amount of fuel for the incoming air charge.

Once the supercharger has worked its magic, the air must be delivered from the supercharger to the engine intake. Although many roots and twin screw superchargers bolt directly to the manifold, most centrifugal superchargers require an extra tube called a Discharge Tube to carry the air to the intake through the throttle body. This tube will normally be mandrel bent to minimize restrictions.

The Bypass Valve
Compressor surge is a problem that affects most superchargers and develops when the supercharger is creating boost, but the throttle shaft is closed. Although not a problem on some low-boost (5psi or less) applications This condition can occur under deceleration or while shifting between gears, and can cause the car to sputter and chirp. Under surge, the compressor forces air into the closed throttle body until the pressure inside the throttle body is higher than the amount of pressure being created by the supercharger, and the air tries to pop backward through the supercharger. At that point, pressure is released inside the throttle body and the compressor forces air back through the supercharger and into the throttle body, which again has nowhere to go, and the process repeats. While surge normally is not highly damaging to the engine it is certainly annoying and can cause damage with time. To eliminate these problems under surge conditions, a bypass valve (sometimes called an anti-surge valve) is used to release the excess pressure. The bypass valve is actuated using intake manifold vaccuum, which opens the vent valve and releases pressure in the air-intake. Air is either released into the atmoshpere (blowoff valve) or recirculated back through the supercharger compressor (bypass valve).

The Intercooler / Aftercooler
Some supercharger systems include an aftercooler (more commonly called an "intercooler"). The purpose of the intercooler is to remove heat from the air to create a cooler, more densely packed air charge - more on this in Let's Talk Intercoolers, and Aftercooling - Vortech Style. Although the intercooler is not necessary on most street applications, its performance becomes increasingly important on higher-output systems (with correspondingly higher charge temperatures). The intercooler can be compared to a automotive radiator, only instead of cooling water or coolant, the intercooler cools the air. Air-to-air intercoolers force the air through a large air-cooled finned and fluted core, normally mounted in front of the car's radiator. Air-to-water intercoolers force the incoming air charge through a much smaller finned and fluted heat exchanger that is cooled by water. The water is, in turn, cooled by a compact radiator that mounts next to the stock radiator.

The two main purposes of the intercooler are 1. to allow more boost on a given octane level of fuel without detonation, and 2. to help create more power by condensing the air charge. Thus, intercoolers are very common on high boost applications (10+ psi) and on roots-style superchargers, where discharge temperatures are higher than normal. Most street supercharger systems (5-8psi) do not come standard with intercoolers.

The Fuel System
As increased amounts of air are pumped into the engine with the supercharger, so too must increased amounts of fuel be delivered. This is where the power gains come from. Most stock fuel systems are not up to the task of delivering the increased volumes of fuel demanded by a supercharged engine. Without a proper fuel system, your engine may run lean, detonate, and obviously perform below its potential. Because every engine is different, the fuel system requirements vary greatly with different vehicles and with different supecharger systems. Sometimes larger fuel injectors and a larger fuel pump is required. On some applications, a fuel management unit (FMU) will do the job by restricting the fuel return line to build up fuel pressure. On other applications, additional fuel injectors are mounted to the intake manifold, while on some applciations the stock fuel system works like a charm. Fortunately most supercharger systems include all of the fuel system components necessary to tune the engine to perfection. On some race kits, tuner kits, custom installations, and high output systems, it is up to the engine tuner to determine the engine's fuel requirements and tune the fuel system accordingly.

The Ignition System
The engine's ignition system serves the important role of telling the spark plugs when to fire so the compressed air and fuel is ignited at the exact right time to produce maximum power. Ignition timing can be advanced, causing the spark to fire earlier, or retarded, causing the spark to fire later. Ignition timing is critical not only for performance reasons, but also for engine longevity as it used to eliminate detonation (aka spark knock). With the added air and fuel that is compressed in a supercharged engine, the engine is closer to its detonation threshold. To avoid detonation, many supercharger systems retard the ignition timing, thus reducing maximum cylinder pressures and temperatures, and moving away from the detonation threshold. Because retarding the ignition timing causes a slight loss in power, a higher octane fuel or an intercooler are recommended for optimal performance, both of which allow for more timing without detonation. To ensure a complete and cool burn, high quality, cool heat range irridium spark plugs are also recommended for use on supercharged engines.

The Pulley
All superchargers are driven by a pulley that sits inline with the accessory belt or crank pulley. The size of the supercharger pulley is what regulates the speed at which the supercharger spins. Obviously, a smaller pulley turns the supercharger faster, and vice versa. The pulley is easy to change on all superchargers and is often used to increase (or decrease) the ouput of the supercharger. A simple pulley-swap can equate to huge power gains if the rest of the system is up to the task (in particular the fuel and ignition system).

The Rest
Other components serve self explanatory roles. Mounting brackets obviosly are used to attach the supercharger to the engine in a position such that the pulley can be spun from the accessory belt or an additional supercharger belt. The belt tensioner keeps the belt tight around the supercharger pulley, which is important to avoid slippage, especially on centrifugal superchargers which spin at high RPMs. Hardware, hoses, and fittings are of course necessary to attach the supercharger to the engine, connect the oil and fuel lines, and to install the ignition components.

That rounds out the complete supercharger system. Remember that every supercharger system is designed to meet the specific needs of the engine, given the desired level of output from the supercharger. For this reason, some supercharger systems come with only a few of the components mentioned in this article, while others come with it all. Generally speaking, higher output supercharger systems come with more components to meet the increased volume of air, which is why they cost more than entry level systems. Congratulations if you made it through all three parts of this series - you deserve a gold star and are now a supercharger expert!

Vortech Q&A 3/26/2003 5:00:00 PM

When you think “supercharger” chances are you’re thinking “Vortech”. The term “Vortech” has become almost interchangeable with “supercharger”. It’s what Kleenex is to tissues, what Aspirin is to pain relievers, and what Rollerblade is to inline-skates. One might wonder how this relatively new entry into the centrifugal supercharger industry has become such a dominant force. Is this company worthy of the title “King of Superchargers”? In seeking an answer to this question, we sat down with our Sales Representative at Vortech Engineering. Here’s what we learned:

SuperchargersOnline.com: We’ve heard you have a $6 million machine that basically cuts a supercharger out of solid block of aluminum. What’s it all about?

Vortech: Our machinery is state of the art. Our machine shop is 10,000sq feet. Our super chargers are actually cast in a foundry. This rough casting then comes in and needs to be whittled into the pieces that make up a supercharger. The machines can automatically grab a pallet of material load it, machine it, and remove it at the end. All small parts are machined from bar stock(literally a solid chunk of aluminum).

SCOL: Last year, Vortech Engineering bought Paxton Automotive. What is the difference between these two companies today?

V: Paxton has been around since the 50’s and was factory equipment on the Studebaker, Avanti and a factory option on many vehicles such as the Shelby GT350. Paxton has always and will always stay in the mustang market, but now they are branching into the Dodge/Chrysler market. You will also see Paxton on high-end vehicles such as the Cadillac Escalade, GMC Denali and the Hummer H2. Vortech will stay focused on the Ford and Chevy car and truck market. 

SCOL: Did you have anything to do with the designing the GM Vortec engines?

V: The engine company is Vortec we are Vortech. There is no relation or association, except we can add a lot of power to a Vortec with a Vortech!

SCOL: How does Vortech decide which new vehicles are worthy of a Vortech supercharger, and which ones get left in the dust? 

V: All of our systems are built based on demand. Once marketing and sales identifies the demand, a vehicle is brought in for evaluation. If a system can be designed, they then determine if it is safe for the motor and can be produced for a reasonable price. If so, they then move on to the R&D process.

SCOL: Why should I choose a Vortech supercharger over a competing brand?

V: In the last 13 years Vortech has become the leader in the centrifugal supercharger industry. Simply pick up any issue of a mustang magazine and you will see nearly all features have Vortech’s on them. We have accomplished this by offering products that are well engineered, triple checked, and still offered at a competitive price. It’s steps like theses that have excelled Vortech to the top. The amount of science and engineering in every unit is what make the Vortech products work better and last longer then any other. This is why Vortech makes over 2 dozen different chargers.

SCOL: If I install a Vortech supercharger on my car / truck, how will it affect my gas mileage?

V: This is a common question. It depends on the driver. If you can control you speed urge it’s possible to get better mileage because you are not making the engine work as hard to produce the same amount of power. However, if you are a speed racer, you have to remember it takes fuel to make power. 

SCOL: What’s the difference between all these head units that Vortech makes – are some for racing and some for the street?

V: Vortech makes over 25 head units and nearly all are dual purpose. Each unit has different flow and pressure characteristics. Vortech uses a SAE approved test stand to map each compressor. We then look at the characteristics of the engine and see which unit will work best. There is always a right tool for the job. You cannot simply assume only a few chargers can meet the needs of all types of engines. 

SCOL: What’s the difference between an S-Trim head unit and an SQ S-Trim head unit, and which one should is the best for a daily driver? 

V: The S-trim head unit is and the SQ-trim head unit both use the same impeller and the same volute. The only difference lies in the gear case. The SQ-trim uses a helical style gear and is virtually silent. The S-trim unit uses a straight cut spur gear and has a pronounced gear whine at idle speeds. However, at driving speeds this charger produces a subtle near silent whistle. 

SCOL: How many miles of trouble free driving is a Vortech supercharger good for before it needs to be rebuilt or replaced?

V: Our chargers typically go 100,000 miles before the slightest maintenance or check-up is needed. One particular customer passed 250,000 miles on his first charger and is approaching 200,000 after his rebuild. 

SCOL: Many people think that ProCharger’s self-lubricated compressor design is superior to Vortech’s, which requires tapping of the oil pan for lubrication. Why doesn’t Vortech switch to a self-contained setup? 

V: 5+ quarts of filtered cleaned engine oil is better suited to act as a lubricant and provide cooling for the charger then a small pool of 6 ounces. The Vortech method requires a little more work initially but requires virtually no maintenance. The competitors require costly and often labor intensive oil changes for the unit itself. 

SCOL: Vortech intercoolers utilize a water sump and radiator to cool the incoming air charge instead of the more common air-air intercoolers we see on Paxton and ProCharger kits. What are the benefits and drawbacks of this intercooler design?

V: Water is nine (9) times more effective for cooling then air is. Think of a stove hot frying pan. Frantically wave it in the air, or shove it in a sink of water. Water is a much better cooling medium. The other benefit of the more effective charge cooler is that it can be smaller. Air-to-air coolers typically loose 2-3psi of boost because of the amount of air it takes to fill the large core and the piping all the way down to the bumper then up to the throttle body. Air-to-water is much more direct and typically looses less then one-forth (1/4) of a psi. 

SCOL: Vortech makes many truck and SUV supercharger systems these days. For heavy vehicles, many people think that a twin-screw or roots supercharger is the way to go. Can a Vortech centrifugal supercharger really compete with a roots or twin screw in the low to mid-range power portions of the power band? 

V: A centrifugal supercharger has a linear power band. The higher the RPM the more boost and power. It works and drives much like a stock vehicle with a much bigger motor. The centrifugal blower is usually 20+% more efficient then a roots or screw type charger. This allows for more boost and a lower discharge temperature. Lower temperatures mean it’s less prone to denotation and will have better power and throttle response. 

SCOL: Why are intercoolers only available for some supercharger systems and not for others? 

V: A charge cooler is not necessary on our systems (with exception to vehicles with a compression ration of 11:1 or higher) because the chargers have a high adiabatic efficiency. This yields a higher flow at a lower temperature. This lowers the detonation threshold thus allowing more boost without a charge cooler then our competitors. However, when a charge cooler is added even more power can be obtained. 

SCOL: If I already have a Vortech supercharger, and I’m looking to get more boost out of it, can I change to a smaller pulley and run it, or do I have to get a new supercharger and/or make other changes? 

V: You can simply swap a smaller pulley in place of the stock one to achieve the desired boost level. You can also upgrade the compressor trim. 

SCOL: What other modifications are recommended to use in conjunction with a Vortech supercharger? Which ones are NOT recommended? 

V: No changes are necessary in any application. However upgrading the clutch can help to manage the increased horsepower. We do not recommend the use of power pulleys because they can change the belt routing and speed of the accessories. 

SCOL: Does Vortech exaggerate its dyno figures, or are the numbers pretty much accurate? Are these things really making around 50% more power?

V: Our numbers are based on a percentage increase on a stock vehicle. All rated numbers are flywheel and are typically conservative. 

SCOL: For $3000 - $4000, what an average Vortech supercharger costs, I could get an array of other modifications for my car / truck. Why should I drop such a huge amount of money on just one modification? 

V: You can spend thousands on pulleys, filters, chips, headers etc… However all those small parts still will not give you the up to 75% horsepower increase that a supercharger system can deliver on a totally stock vehicle. 

SCOL: What new systems can we look for out of Vortech in the upcoming months? 

V: We are expanding our C5 Corvette line and you can also expect to see a Focus and Focus SVT system soon. 

SCOL: Are there any questions you wished we asked, but didn’t? 

V: NO, I think you asked enough!

FEATURES :: SUPERCHARGER TECH As Technical As Twin-Screws Get 3/3/2003 6:45:00 PM

Welcome to the 2nd installment from Aleksandar Subic - Associate Professor at RMIT University in Australia. At RMIT, they are making giant strides to understand, and improve upon, the intricacy and technology of the twin-screw supercharger. The following information has been graciously offered by the RMIT team, to our online community, so that we may all benefit from the knowledge that their findings provide. Enjoy!

Interest in the use of superchargers as a power-boosting device for automotive engines has increased greatly in recent years. The interest is driven by environmental (lower emissions), cost (reduced engine size & weight), and marketing (higher engine output power) requirements. The twin-screw supercharger is most suited for high engine power automotive applications and has been used by some manufacturers, e.g. Mazda’s Miller engine, as well as an aftermarket add-on kit.

The twin-screw supercharger uses two intermeshing rotors to trap and move air between the inlet and outlet manifolds compressing the air charge along the flow pathway. The rotors rotate at very high speeds (10000 – 20000 RPM). The automotive twin screw supercharger is of a dry type (i.e. - No lubrication is applied between the rotors. Lubrication is, however, used on the internal gears that spin the rotors). The rotors must not come in contact with each other in order to avoid excessive friction and heat generation. This would lead to rotor seizure and would damage the supercharger. The rotors are manufactured with very tight clearances to overcome the contact problem (i.e. - Nominal clearances around 0.2 mm and reaching less than 0.08 mm at the closest point between the rotors).

The gaps between the meshing rotors lead to axial and radial air leakage during the compression process, known as parasitic losses. This reduces the net output and efficiency of the twin-screw supercharger especially at partial load operation. There is a need to properly identify and quantify the locations with the highest leakage losses and focus on improving the sealing action of the intermeshing rotors at such locations. The use of advanced Computational Fluid Dynamics (CFD) simulation and visualization would give a better insight into the fluid dynamics in the flow pathways and the leakage mechanism between the rotors.

The fluid flow inside the supercharger is highly turbulent and can reach sonic speed across the leakage pathways leading to localized chocking or even supersonic flow conditions. Proper CFD modeling of the highly turbulent flow along the rotor flow pathway, and very small gaps formed by the intermeshing rotors, requires the use of a very fine grid with hundreds of thousands of elements. The high speed of the rotors results in time increment restrictions in the CFD simulation. This translates into significant computation power and storage requirements.

Satisfying these requirements can only be achieved by using a High Performance Computing (HPC) facility with advanced parallel processing and storage capabilities. Current research at RMIT University (Dr. Bassam Abu-Hijleh, Associate Professor Jiyuan Tu, Associate Professor Aleksandar Subic) focuses on the use of state of the art CFD computation for the simulation and optimization of twin-crew supercharger as well as to provide guidelines for future twin-screw designs with enhanced efficiency and performance over the entire operating regime of the engine. To this effect, the current research project has at its disposal a wide range of computing facilities ranging from standard Pentium 4 PCs and a dedicated dual Xeon CPU workstation all the way to a 128 CPU Compaq Alphasever supercomputer courtesy of the Victorian Partnership for Advanced Computing (VPAC).

The project is divided into the CFD simulation of several simplified models of the twin-screw supercharger before attempting to model the operation of an actual twin-screw supercharger. Each model is designed to gain experience in one or more of the many specific areas and expertise needed to be able to fulfil the final goal of full 3D dynamic simulation and visualization of the airflow within a twin-screw supercharger. Models successfully simulated include a stationary 2D version of the twin-screw rotors (see Figure 1 below). This model was used to determine the proper CFD parameters needed to be able to simulate the high-speed compressible flow in the leakage pathways.

| Fig. 1a - Velocity contours (m/s) in a 2D twin-screw supercharger. Operating pressure ratio of 3.34 (i.e. - 237 kPa/34.4psi pressure boost). Flow from left to right. | Fig. 1b - Close-up of the velocity contours in the circular region shown in Fig. 1a. |
| --- | --- |

A model of a generic 2D two-rotor dynamic configuration was also simulated successfully (see Figure 2). This model was used to gain the required expertise of using a dynamically changing mesh, the latest in CFD capability available from FLUENT software. In dynamic CFD simulation of a twin-screw supercharger, the mesh used to simulate the fluid flow needs to “adapt” to the changes in the geometry that result from the rotors motion from one time step to another.

| Fig. 2a - Dynamically changing mesh with rotors’ motion. Right click here to download movie. (16.9 MB) | Fig. 2b - Pressure contours variation with rotors’ motion. (Note the pressure waves travelling in the inlet and outlet manifolds). Right click here to download movie. (7.54 MB) | Fig. 2c - Velocity contours variation with rotors’ motion. Right click here to download movie. (8.87 MB) |
| --- | --- | --- |

This is not an easy thing to accomplish, as it requires extensive experience in initial mesh generation and is extremely computer intensive. The experience gained from these two simplified models will be combined, currently underway, and then advanced into 3D before working on the full 3D dynamic simulation of a twin-screw supercharger early in 2003.

Stay tuned for more!

FEATURES :: SUPERCHARGER TECH Bye Bye 14.7 psi! 3/2/2003 9:30:00 PM

Did you know that the reason your engine makes MORE power with a supercharger is the same reason it makes LESS power as you gain elevation? You lose power as you gain elevation simply because the air is LESS densely packed with air molecules. Conversely, a supercharger packs in air that is MORE densely packed with air molecules. In this article we take a look at air pressure and how important it is to engine performance.

It's impossible to understand how a supercharger works unless you understand what air pressure is. You probably already know that a supercharger increases the air pressure in the intake manifold, but let's start by considering air pressure in our natural environment - standard atmospheric pressure.

Atmospheric Pressure - 14.7 psi

When we think of air, we normally think of a weightless gas that fills our atmosphere. Reality, however, is much different. A fish likely thinks that water is weightless, because there is no force that he feels pushing down from above - the pressure comes from all directions, so he does not feel the weight of the water. Like little fish swimming in the water, humans walk through the atmosphere that seems weightless, but is indeed very heavy.

The atmosphere is the mass of air that surrounds Earth. It extends upward from Earth's surface approximately 500 miles in total. The Troposphere is the lowest and densest layer of the atmosphere where all weather takes place - it contains 99% of all water vapor molecules and 75% of the atmosphere's total mass. The Troposphere extends 5 to 10 miles above Earth's surface depending on where you measure from (10 miles at the equator, 5-7 miles at the poles). Beyond this layer are the Stratosphere (within which lies the Ozone layer), the Mesosphere, and finally the Ionosphere. All of these layers of atmosphere contain air molecules, particles, and atmos - all of which have mass that is attracted to Earth by gravity.

If you were to measure a column of air 1-inch square extending to the top of the atmosphere, this column of air would weigh approximately 14.7 pounds at sea level. Thus, atmospheric pressure at sea level is around 14.7 psi. If you measured from a point above sea level, you would notice that the weight of the column of air would decrease by approximately one pound for every 2343 feet of elevation gain. The reason for this decrease is because at higher elevations, there are fewer molecules of air in the column being pulled to the Earth, which lightens the column proportionally. This concept is easy to understand if you think of a stack of books. The more books there are, the more pressure there is on the ones at the bottom. The books at the top feel very little pressure. Because air compresses under the weight of the molecules from above, the air molecules become more densely packed as the column becomes taller. The column is tallest when elevation is the lowest, so there are more air molecules at lower elevations.

Back to Superchargers

You may be wondering what all this has to do with superchargers. So far, we have learned that, at lower elevations, there are more air molecules. We also know that the internal combustion engine works by igniting a mixture of air and fuel. To make more power, we need to burn more air and more fuel. We can do this using a bigger engine, or by simply going down in elevation, where there are more air molecules. However, we can only go so low before we hit the ocean or the valley floor, and Mother Nature taps out at around 14.7 psi of pressure. Now if we could only make our column of air taller, we could have more air molecules and make more power!

If you haven't already figured it out, this is where the supercharger steps in to take charge. A supercharger is simply an air pump or compressor that crams more air molecules into the engine's intake than would fit under normal atmospheric conditions. At 7psi of boost, manifold pressure at sea level would be around 21.7 psi (14.7 + 7 = 21.7), meaning we would have almost 50% more air, and can make almost 50% more horsepower. This is why we say, "it's better under pressure".

FEATURES :: SUPERCHARGER TECH Superchargers A-Z (Part 2 of 3) 2/26/2003 12:43:00 AM

Welcome to part 2 of "Superchargers A-Z". If you haven't already read part 1, of this series, you may want to start there.

Some of you may have recognized in part 1 of this series that in the early days of supercharging, there are three types of superchargers - roots, twin-screw, and centrifugal. You may already be familiar with these buzz-words, but most people don't understand how each technology differs. Before buying a supercharger, you should familiarize yourself with how each type of supercharger works. Each has its own set of advantages and disadvantages that may make it ideal - or not - for your performance needs. Today we take a technical look at the technology behind each type of supercharger.

First lets begin with some basics. There are many components that go into making a complete supercharger system - mounting brackets, ignition controller, fuel pump, etc. In this article we look at only one component of a supercharger system - the supercharger itself (sometimes called a "head unit", "compressor", or "blower"). All superchargers, except turbochargers, are driven via a pulley that is connected either to the engine's accessory belt, or to its own belt that goes directly to a crank pulley. This is where the similarities between the different supercharger technologies end.

The Roots Supercharger (aka "blower")
The roots supercharger was originally designed as an air moving device for industrial buildings. The roots supercharger features two counter-rotating lobes that trap air from the intake side of the supercharger (normally at the back of the supercharger), move it around the outside casing of the lobes, and out the bottom of the supercharger through an outlet / discharge port. Like the twin screw supercharger, the roots is a "positive displacement" aka "fixed displacement" supercharger, meaning that it moves a fixed volume of air per rotation. Notwithstanding minor amounts of air-leak at low rpms, the roots supercharger cannot flow backwards like a centrifugal supercharger, and is thus nearly as efficient in its ability to pump air at low rpms as it is at high rpms. What this means is that roots superchargers are very capable of making large amounts of boost even when engine rpms are very low. This makes for great low-end and midrange power, and also makes them great for trucks and towing vehicles. The roots is also self lubricated, and is the simplest of the supercharger designs, meaning it is reasonably priced and very reliable. This is why roots superchargers have been the choice of GM, Ford, Mercedes, and Toyota for OE applications.

The only real disadvantage to the roots supercharger is that it creates a lot of heat. There are two reasons for this. First, the roots supercharger does not compress air - it only moves from the intake port to the discharge port (i.e. it is the only supercharger design with no internal compression ratio). All of the compression is done in the intake manifold. Laws of thermodynamics kick in in favor of supercharger designs with an internal compression ratio (centrifugal and twin screw) because they do less work on the incoming air charge. We will leave the mathematics of this phenomenon to a later (much more boring) discussion. Another reason roots superchargers create higher amounts of heat is because they tend to carry some of the compressed air in the intake back into the supercharger because it gets trapped by the rotating lobes that are exposed to the hotter air in the intake manifold.

| A roots supercharger ("blower"). | Want to know why a roots supercharger creates more heat than a centrifugal or twin screw? Calculate the amount of work each does on the incoming air charge and measure the area underneath the curve on the Pressure Volume Graph. |
| --- | --- |

The Twin Screw Supercharger
The twin screw supercharger at first glance appears to look similar to a roots supercharger both inside and out. The two technologies are indeed similar, however there are significant differences. At the heart of the twin-screw supercharger are two rotors, or "screws" that rotate towards each other. The rotors mesh together and draw air from the back of the supercharger. The twisting rotors move the air to the front of the supercharger, while compressing the air before discharging through a port at or near the front of the supercharger.

Because the compression is done inside the supercharger, this design produces less heat than a roots supercharger - in fact, it is almost as thermally efficient as a centrifugal design. Like the roots design, the twin-screw is a fixed displacement supercharger (meaning that it pumps a fixed volume of air per revolution), and because the tolerances between the rotating screws are very tight, its ability to create boost at low rpms is unparalleled. These characteristics make it ideal for trucks and towing vehicles, where low to mid range power is primary in importance. Another important advantage of the twin screw compressor is its reliability. Unlike a roots supercharger, the rotors in a twin screw supercharger do not actually touch, so there are virtually no wearing parts. For this reason, twin screw compressors are commonly used to pressurize cabins in passenger aircraft. Like roots superchargers, twin screw superchargers are self lubricated and do not tap into the engine's oil supply.

One disadvantage of the twin screw design is that, because it has an internal compression ratio, the twin screw is compressing air even when it is not sending boost to the engine (i.e. under cruising or deceleration). An internal bypass valve releases the pressurized air, but because it takes work to pressurize the air in the first place, the twin screw supercharger draws more power from the engine than while not under boost. Like the roots, the throttle body must be placed before the compressor because it is a fixed displacement supercharger.

| A cutaway view of a twin screw supercharger. | Airflow through a twin screw supercharger. |
| --- | --- |

The Centrifugal Supercharger
Although the centrifugal supercharger is founded on a technology much newer than either the roots or the twin screw, it was the first supercharger to be successfully applied to automotive applications. Unlike the roots, the centrifugal supercharger is NOT a positive displacement / fixed displacement supercharger because it does not move a fixed volume of air per revolution. The centrifugal supercharger essentially operates like a high speed fan propeller / impeller, sucking air into the center of the supercharger and pushing it to the outside of the rapidly spinning (40,000 + rpm) impeller blades. The air naturally travels to the outside of the blades because of its centrifugal force created by its rotating inertia. At the outside of the blades, a "scroll" is waiting to catch the air molecules. Just before entering the scroll, the air molecules are forced to travel through a venturi, which creates the internal compression. As the air travels around the scroll, the diameter of the scroll increases, which slows the velocity of the air, but further increases its pressure.

The centrifugal supercharger enjoys several advantageous characteristics that make it the most popular supercharger design in the aftermarket world. First, it is simple and reliable because it has very few moving parts - just a few gears and the impeller. Second, the centrifugal supercharger produces very little heat because of its internal compression ratio. It is also small in size and very versatile because it can "free-wheel" and allow the engine to suck air through it or even flow air backwards. For this reason it can be placed anywhere in the intake tract - it can even "blow through" the throttle body, meaning it can be mounted nearly anywhere. It is also the most thermally efficient supercharger, meaning that it produces the lowest discharge temperature.

The only significant disadvantage of the centrifugal supercharger is that it must be spinning at a relatively high speed before it begins to make a significant amount of boost. For this reason, it is not helpful in creating boost (and power) at low engine rpms. Normally the supercharger only begins to create boost at around 3000 rpm, and the boost curve gradually and increasingly rises with engine RPM. Many centrifugal superchargers do not have a self-lubricating oil system, and draw oil from the engine's oil supply. The disadvantage to this is that you must tap the oil pan for the oil return line. However, in doing so, the supercharger becomes virtually maintenance free. Some manufacturers make a "self-contained" centrifugal supercharger that is self-lubricated like roots and twin screw superchargers.

| A centrifugal supercharger. | Airflow through a centrifugal supercharger. |
| --- | --- |

The Turbocharger
You may be wondering where the turbocharger fits in to this equation. Technically, a turbocharger IS a type of supercharger - one that is driven by exhaust gasses rather than from a pulley that draws power from the engine's crank. Because we have covered this topic in depth in our Turbos vs. Superchargers article, we will not re-examine the differences again here. Because the turbocharger relies on a technology substantially different from the three traditional supercharger technologies discussed above, it is beyond the scope of this article.

That's it for part 2 of the series - next time we'll pull everything together and discuss what goes into making a complete supercharger system, and how the supercharger works in conjunction with the engine.