The Mystery of 'Octane' Finally Uncovered

What is octane? The concept of octane is peculiar to many people, including myself. I can't see it, I can't hold it, I can't buy it by itself, but I know I know I pay extra for it if I want more than 87 of it in my gas. Today we take an in-depth look at what octane is, and what significance a gasoline's octane rating has with regards to a supercharged engine.

The Simple Definition

An octane rating (87 vs. 89 vs 92, etc.) is a measure of a gasoline's ability to resist detonation, which manifests itself in a pinging or "knocking" noise. Higher numbers indicate that the fuel can be compressed to a higher level before detonation / knock occurs in an engine, which occurs when. As described in "Detonation, Knock, and Pre-Ignition 101", detonation / knock occurs when air and fuel that is ahead of the combustion flame front ignites before the flame front arrives.

The Complicated Definition

Octane is actually more than just a rating - it is a hydrocarbon just like methane (single carbon atom), propane (three carbon atoms), butane (five carbon atoms), and heptane (seven carbon atoms). Octane (C8H18) is a hydrocarbon with eight carbon atoms and eighteen hydrogen atoms. 100% octane fuel is remarkably resilient to compression (i.e. it does not combust when compressed) and is thus resilient to detonation / knock. This resilience is derived from the branching of octane's carbon chain (see figure). Because of the nature of octane as being resilient to detonation, all fuels are compared to 100% octane as a benchmark fuel, from which an "octane rating"can be obtained. Heptane, a hydrocarbon with seven carbon atoms, compresses very poorly and spontaneously combusts even under small amounts of compression. In other words, Heptane's behavior when compressed is diametrically opposed to Octane's behavior under the same conditions. For this reason, Heptane (which has an octane rating of zero) is the other benchmark fuel used in the octane rating system to determine a fuel's octane rating. A fuel that spontaneously combusts (knocks) under the same amount of compression as a fuel composed of 87% octane and 13% Heptane would have an octane rating of 87. This is not to say that 87 octane gasoline is made up of 87% octane and 13% heptane, rather that the 87 octane gasoline "knocks" in a laboratory knock engine at the same compression ratio as a fuel composed of 87% octane and 13% heptane.

The coomposition of an octane hydrocarbon.

Unfortunately, it gets even more complicated. Because various fuels respond differently under varying engine loads, a gasoline may get a different octane rating on a free running engine and one under load. For this reason, the octane rating label that we see at the pump (monitored by the U.S. Cost of Living Council) is actually an average of two octane ratings - the motor method rating (where the engine is run under a load) and a research method rating (where the engine is run freely). The formula used to get the CLC Octane number on gas pumps in the United States is thus: (Motor Octane Number + Research Octane Number) / 2.

What's the benefit of higher octane?

Higher octane fuel has only one beneficial feature - it allows an engine to run at higher temperatures with more advanced ignition timing under higher levels of compression witout detonating / knocking. Higher octane fuel does NOT have more potential energy and will not make an engine perform better unless that engine is knocking. On modern engines with knock sensors, higher octane fuel may make the engine run better if the knock sensors are retarding the ignition timing, which hinders performance. High octane fuel does not burn cleaner, it does not clean your engine, it does not increase horsepower or torque (unless you are experiencing knock), it does not smell better, it does not increase fuel economy (unless you are experiencing knock) and is not better for the environment. If you buy higher octane fuels for any of the above reasons, STOP!

When should I switch to a higher octane fuel?

First off, never run lower octane fuel than is recommended by the vehicle's manufacturer. If the vehicle manufacturer recommends 89 octane gasoline, this means that the engine has been tuned to perform optimally without detonation on 89 octane fuel. Once you've done some modifications to your engine, the manufacturer's recommended gasoline may no longer suffice. Obviously, if you can hear detonation inside your engine in the form of pinging or "knocking", try a higher octane fuel. You will also need to run a premium grade fuel (91+ octane) if you have a supercharger, turbocharger, or if you have an ignition programmer that advances your ignition timing.

Why is higher octane fuel more expensive?

Higher octane fuels are more expensive because they must go through more refining steps that increases the octane rating. These additional steps do not make the fuel better in any other way.

How is it possible to have 100+ octane gasoline?

There are some fuels that are even more resilient to compression than 100% octane. Some additives, like tetraethyl lead, increase the gasoline's ability to operate without knock. Some racing and airplane fuels have octane ratings of 110+!

Detonation, Knock, and Pre-Ignition 101 8/4/2003 1:59:00 PM

As you probably have already figured out, detonation (aka "knock") is a big issue in the world of forced induction. You probably know that detonation is a bad thing, and that by adding a supercharger (or any forced induction power adder), you must take additional measures to avoid detonation, especially if your engine has other modifications. Normally the simple solution to stop detonation is to run higher octane fuel... but before we get ahead of ourselves, let's start from the beginning.

What is detonation / knock?

Under normal conditions, the combusting air and fuel mixture inside the combustion chamber ignites in a controlled manner. The mixture is ignited by the spark, normally in the center of the cylinder, and a flame front moves from the spark towards the outside of the cylinder in a contolled burn. Detonation occurs when air and fuel that is ahead of the flame front ignites before the flame front arrives because it becomes overheated. Under these conditions, the combustion becomes uncontrolled and sporadic and often produces a pinging noise, or a "knock" noise when the conditions become worse.

So far, detonation sounds cool... why is it bad?

Detonation is definitely not cool. Detonation causes sudden pressure changes in the cylinder, and extreme temperature spikes that can be very damaging on engine pistons, rings, rods, gaskets, bearings, and even the cylinder heads. Even the best engine components cannot withstand severe detonation for more than a few seconds at a time. More severe detonation obviously leads to more severe forms of engine damage. If there is enough heat and pressure in the combustion chamber, detonation can begin to occur before the spark plug even fires, which would normally initiate the combustion. Under these circumstances, known as "pre-ignition", the piston may be travelling up towards a wave of compressed, exploding gas. These are the worst kinds of detonation conditions, and can bend con-rods and destroy pistons.

What causes detonation?

Detonation occurs when several conditions / factors inside the combustion chamber exist at the same time. Increased compression, high temperatures, lean fuel/air mixture, advanced ignition timing, and lower octane fuels are all factors that PROMOTE detonation conditions. The good news is that, because there are so many factors in play, you can always find a way to eliminate detonation if it exists.

So, where do superchargers fit in?

A supercharger increases the amount of air inside the combustion chamber (see "Bye Bye 14.7 psi"), which in turn increases the compression inside the combustion chamber. Along with increased compression comes higher temperatures and higher pressures, which as we know, tend to increase the chances that some form of detonation will occur. In order to compensate for the increased compression and heat, we must change one or more of the other factors / conditions to move us away from our detonation threshhold. Tuning the supercharger system to the engine in this way for maximum performance without detonation is something that supercharger manufactuers do so, chances are, you won't have to worry about it unless you do other modifications to your engine that place you closer to your detonation threshhold.

How do I get rid of it?

The two most common tricks used by supercharger manufactuers and engine tuners looking to obtain maximum performance without detonation is 1. use higher octane fuel, and 2. retard the ignition timing.

Higher octane fuel burns more controllably and is not as likely to combust before the flame front. This is why racing engines use 100+ octane gasoline. The ONLY benefit of racing gasoline is that it moves you away from the detonation threshhold, which allows you to be more aggressive with power producing factors - i.e. raise compression, advance timing, etc. This is why you'll be disappointed if you put racing gasoline in your mom's bone-stock '82 Toyota Cressida thinking you'll turn it into a race car. If you don't have detonation, the increased octane will do you no good. For cars designed for daily street driving, you obviously won't want to fill up with 100+ octane fuel every week at the tune of 5 bucks a gallon. This is why supercharger manufactuers tune their supercharger systems to run properly without detonation on 91 octane fuel - aka "premium" at your local gas station (in some states premium gasoline is around 93 octane).

Retarding the ignition timing will delay the timing of the spark, which also moves you away from your detonation threshhold. Most popular "power programmers" or "chips" increase engine power by advancing the ignition timing, and requiring you to run a higher octane fuel to avoid detonation. These work great, except the advanced ignition timing is NOT compatible with most superchargers, unless you're happy to run 100 octane fuel. In fact, many supercharger systems include an "ignition boost retard" that retards the ignition timing when it senses boost from the supercharger. This allows you to maintain stock performance while not under boost, yet still remain safe while the supercharger is making its boost (and power).

Another way to avoid detonation is to cool the incoming air charge to lower the temperature inside the combustion chamber. On a supercharged application, this task can be handled by an intercooler (see "Let's Talk Intercoolers") or by a water injection system (less common). The intercooler takes the incoming air charge and passes it over a series of air-cooled or water-cooled fins and ducts, thus cooling the air in the same way that a radiator cools your engine's coolant. Intercoolers are thus very popular in higher output supercharger systems, where detonation becomes more of a problem. Often times, the intercooler allows you to run more boost and also allows you to eliminate the ignition boost retard, meaning you'll notice increased performance, and still experience no detonation. Another way to lower the temperature of the combusting air and fuel is to run cooler heat range spark plugs. Many supercharger manufacturers will recommend cooler plugs for you supercharged engine.

Because lean condition (fuel starvation) also contributes to detonation, it is important to make sure that the fuel system (pump, injectors, etc.) is capable of delivering the increased fuel requirements of the supercharged engine. Often times, an otherwise perfectly tuned engine will experience detonation just because the fuel pump can't deliver enough fuel to the engine. Upgrading certain fuel components is almost always necessary when supercharging an engine. Most supercharger systems normally include the upgraded fuel components if they are necessary. If you are installing a supercharger on an engine with other modifications, make sure you consider the additional fuel requirements and compensate with larger injectors and / or a bigger fuel pump.

Some modern vehicles come with "knock sensors" that listen for detonation, and automatically retard the ignition timing to eliminate detonation. Although these devices are effective in preventing engine damage, they are not tuned for performance, so you should not rely on the knock sensors and expect your engine to run its best.

Conclusion

Altough detonation can be potentially damaging to an engine, a simple understanding of what it is, and what causes it, will help you stay away from your detonation threshhold. Pay attention to "knock" and pinging noises that come from your engine becuase they could indicate detonation inside the combustion chamber and should be dealt with immediately. If you're looking for a new supercharger system, don't worry too much about detonation - the manufacturers have designed the system for use on your stock engine, and if you follow the manufactuer's fuel recommendations, you will not have a detonation problem. If you ever do notice detonation, perhaps from bad (low octane) gasoline or extremely high air temperatures, just drive with a light foot until you are able to resolve the cause of the problem.